# WH-700MSA Air Pulse Parts Cleaner

Air Pulse cleaning creates bubbles and they implode violently causing cavitation effect. The effect creates an intense scrubbing action on the surface of the item being cleaned.

## Installation Before Usage

- Unpack the unit from carton and inspect for damage.

- Connect air inlet to standard air supply.

## Detergent Filling Procedure

- Make sure the depletion cap on the side of the unit is tightly secured.

- Fill in recommended detergent (water soluble cleaning agent). The tank volume is 20 litres.

Note: Do not use Flammable solvents (gasoline, toluene, thinner and etc.)

## Operating Instruction

- Open the lid.

- Make sure sufficient detergent inside the tank. (The object to be cleaned should be submerged under the detergent).

- The detergent can be recycled and used for next operation.

- Close the lid

- Turn on the air switch on the air inlet to start the cleaning. The air supply should be between 6~8 bars.

- Alternatively the object could be soaked in the solvent to loose up dirt/carbon/grime on the surface before cleaning.

- The cleaning time varies depending on object size and cleanness.

- Disconnect the air supply and close the lid when finished.